SentryAgent

AI-Powered Local IoT Home Security System

Project Report

IoT Systems — University of Alabama

June 2026

# Abstract

SentryAgent is a fully local, AI-powered home security system that integrates IoT edge hardware with on-device large language model (LLM) reasoning. Unlike commercial security platforms that depend on cloud APIs and third-party data processing, SentryAgent processes all sensor data and AI inference on a local network. A Raspberry Pi collects motion, sound, and temperature data and streams live camera footage; a laptop-hosted Python backend classifies events with rule-based logic and escalates suspicious activity to a locally running Ollama LLM; and a Flutter mobile application provides a real-time dashboard, camera feed, reasoning log, event history, and conversational agent interface. The system demonstrates that privacy-preserving, intelligent home security is achievable without cloud dependency, subscription fees, or data exposure.

# Table of Contents

1.  Introduction

2.  System Architecture

3.  Hardware Components

4.  Backend Implementation

5.  Mobile Application

6.  Communication Protocol (MQTT)

7.  AI & Machine Learning Integration

8.  System Integration & Data Flow

9.  Testing & Evaluation

10.  Results & Demo

11.  Limitations & Future Work

12.  Conclusion

# 1. Introduction

## 1.1 Problem Statement

Modern home security solutions — Ring, Nest, SimpliSafe — rely heavily on cloud infrastructure for video storage, event analysis, and mobile push notifications. This dependency introduces multiple concerns: user privacy (footage stored on third-party servers), latency (round-trip to cloud before an alert fires), monthly subscription costs, and complete system failure during internet outages. Furthermore, these systems apply only shallow rule-based detection (motion zones, sound thresholds) without contextual reasoning — they cannot distinguish a family member arriving home at midnight from an actual intruder.

## 1.2 Proposed Solution

SentryAgent addresses these limitations by running entirely on the local network. Sensor data flows from a Raspberry Pi over MQTT to a Python backend that applies a two-layer detection strategy: a fast rule-based classifier filters obvious benign or obvious threat events, while a locally hosted LLM (Qwen 2.5 7B) provides contextual reasoning for ambiguous cases. Results are displayed in real time on a Flutter mobile application. No data leaves the home network.

## 1.3 Objectives

Build a real-time IoT pipeline from physical sensors to mobile app using MQTT.

Implement a two-layer threat detection system combining rules and LLM reasoning.

Deploy a local LLM (Ollama) capable of tool-calling for autonomous security decisions.

Develop a polished Flutter mobile app with live camera, charts, chat, and notifications.

Demonstrate the complete system in a university environment using real hardware.

# 2. System Architecture

## 2.1 High-Level Overview

SentryAgent follows a three-tier edge-processing architecture. The edge tier (Raspberry Pi) handles raw sensor acquisition and camera streaming. The processing tier (laptop, Docker) runs the MQTT broker, AI agent, SQLite database, and camera relay. The presentation tier (Flutter mobile app) connects to the processing tier over the local network for live data and control.

## 2.2 Architecture Diagram

The data flow follows this path:

Raspberry Pi reads GPIO sensors (PIR motion, sound, DHT11 temperature/humidity) and publishes combined JSON to MQTT topic  iot/pi/telemetry.

Pi camera module streams JPEG frames over WebSocket to the camera relay service on port 8000.

The Python orchestrator receives telemetry, splits fields into typed SensorReading objects, and passes them to the event classifier.

The classifier applies rule-based logic and produces SecurityEvent objects with threat levels: safe, warning, or alert.

Warning and alert events are queued for LLM evaluation by the Ollama-backed decision engine.

The agent publishes SecurityState (on every telemetry tick, ~2 s), SecurityEvent, and AgentDecision messages to MQTT.

The Flutter app subscribes to these topics and updates all screens in real time.

The user can arm/disarm the system, chat with the agent, and trigger/stop the siren from the app.

## 2.3 Deployment Topology

| Component | Host | Port / Protocol |
| --- | --- | --- |
| Mosquitto MQTT Broker | Laptop (Docker) | 1883 TCP, 9001 WebSocket |
| Python AI Agent | Laptop (Docker) | Internal |
| Camera Relay (FastAPI) | Laptop (Docker) | 8000 WebSocket / HTTP |
| Ollama LLM Server | Laptop (Docker or native) | 11434 HTTP |
| SQLite Database | Laptop (Docker volume) | File I/O |
| Sensor Publisher | Raspberry Pi | MQTT client → 1883 |
| Camera Streamer | Raspberry Pi | WebSocket → 8000 |
| Flutter Mobile App | Android Phone | MQTT + WebSocket client |

# 3. Hardware Components

## 3.1 Raspberry Pi

The Raspberry Pi serves as the IoT edge node. It runs two Python scripts: pi_sensor_publisher.py for sensor data and pi_camera_streamer.py for video streaming. The Pi connects to the laptop over the university Wi-Fi (UA network) or via Ethernet with Internet Connection Sharing.

## 3.2 Sensors

| Sensor | Type | GPIO Pin | Data Published |
| --- | --- | --- | --- |
| PIR Motion Sensor | Passive Infrared | GPIO 22 | motion: Active / Inactive |
| Sound Sensor Module | Digital comparator (LM393) | GPIO 27 | noise: 40-90 dB-scale (pulse-rate mapped) |
| DHT11 | Digital temp/humidity (kernel IIO driver) | GPIO 23 | temperature (degC), humidity (%) |
| Pi Camera Module | MIPI CSI | CSI connector | JPEG frames @ ~10 fps |
| Active Buzzer | Digital out | GPIO 18 | Fires when temp exceeds threshold or via app command |
| Status LEDs | Digital out | GPIO 13/5/19/26/21 | Armed, high-temp, high-humidity, motion, sound indicators |
| Arm/Disarm Button | Digital in (pull-up) | GPIO 6 | Publishes arm/disarm request over MQTT |

## 3.3 Pi Telemetry Format

The Pi publishes a single JSON object every 2 seconds to iot/pi/telemetry:

{"temperature": 23.8, "humidity": 67.0, "motion": "Active", "noise": 51.8, "mode": "Armed", "buzzer": "OFF", "timestamp": 1781120000}

Sound processing: the LM393 module outputs only a binary pulse when the microphone level crosses a potentiometer-set threshold. To obtain a continuous, realistic reading, the publisher counts comparator pulses with a GPIO interrupt callback (so even ~1 ms spikes are never missed) and maps the pulse rate to a 40-90 dB-style scale: 40 in silence, ~55 for a clap, ~70 for conversation, 90 for sustained loud noise. A dedicated sound LED (GPIO 21) lights instantly on detection for on-device feedback.

## 3.4 Network Setup

For the university demo, the laptop shares its UA Wi-Fi connection to the Pi over Ethernet (Internet Connection Sharing). This gives the Pi a stable DHCP IP (192.168.137.x) without requiring WPA2-Enterprise credentials on the Pi itself. The Pi scripts are configured with the laptop LAN IP (192.168.137.1) as the MQTT and camera relay target. The Pi is assigned a permanent static address (192.168.137.38) via a NetworkManager profile, and a crontab @reboot entry launches the sensor publisher and camera streamer automatically at boot, making the demo setup plug-and-play.

# 4. Backend Implementation

## 4.1 Technology Stack

| Technology | Version | Purpose |
| --- | --- | --- |
| Python | 3.12 | Primary backend language |
| aiomqtt | latest | Async MQTT client |
| Pydantic v2 | 2.x | Data validation & serialization |
| aiosqlite | latest | Async SQLite persistence |
| FastAPI + Uvicorn | latest | Camera relay WebSocket server |
| Ollama | latest | Local LLM inference engine |
| Qwen 2.5 7B Instruct | latest | Default reasoning model |
| Jinja2 | latest | LLM prompt templates |
| Docker + Compose | latest | Service orchestration |
| Mosquitto | 2.x | MQTT broker |
| pytest | latest | 74 automated tests |

## 4.2 Module Structure

| Module | Responsibility |
| --- | --- |
| orchestrator.py | Central service — coordinates all MQTT, classifier, LLM queue, state publishing |
| event_classifier.py | Rule-based Layer 1 — converts SensorReading → SecurityEvent with ThreatLevel |
| agent.py | LLM Layer 2 — DecisionEngine with Ollama + tool-calling loop |
| chat.py | ChatService — user conversation with the agent, maintained history |
| tools.py | LLM tool registry — 6 callable tools (query events, trigger siren, etc.) |
| storage.py | Async SQLite — events, decisions, chat, state tables |
| models.py | Pydantic domain models shared across all modules |
| mqtt/bus.py | MqttBus — async pub/sub abstraction with handler registry |
| camera/relay.py | FastAPI WebSocket hub — Pi pushes, app views, MJPEG endpoint |
| llm/ollama.py | OllamaClient — HTTP to /api/chat with native tool calling |
| sensors/mock.py | MockSensorPublisher — simulates Pi without hardware |

## 4.3 Event Classifier (Layer 1)

The classifier applies deterministic rules to convert raw sensor readings into security events. This avoids calling the LLM for every heartbeat — only genuinely interesting events escalate.

| Sensor | Condition | Threat Level |
| --- | --- | --- |
| Motion | Active, system armed, night (22:00–06:00) | ALERT |
| Motion | Active, system armed, daytime | WARNING |
| Motion | Active, system disarmed, night | WARNING |
| Motion | Active, system disarmed, daytime | SAFE |
| Sound | Level >= 80 dB | ALERT |
| Sound | Level 60-79 dB | WARNING |
| Door | Opened while armed | ALERT |
| Temperature | < 5°C or > 38°C | WARNING |

## 4.4 LLM Decision Engine (Layer 2)

When the classifier produces a WARNING or ALERT event, it is placed into a bounded decision queue (max 10). A single-threaded worker processes these serially to avoid overwhelming the LLM. The engine builds a structured prompt from the Jinja2 template event_evaluation.j2 containing: current security state, the triggering event, recent event history, and available tools. The model may call tools iteratively before returning a final AgentDecision with an action.

Available LLM tools:

| Tool | Action |
| --- | --- |
| query_recent_events | Retrieve recent security events from SQLite |
| query_sensor_state | Get current readings for any sensor type |
| acknowledge_event | Mark an event as reviewed (lowers threat score) |
| trigger_siren | Activate or stop the siren on the Pi |
| notify_user | Send a priority message to the mobile app |
| mute_sensor | Temporarily suppress a noisy sensor |

## 4.5 Persistence

SQLite in WAL (Write-Ahead Logging) mode stores all events, agent decisions, chat messages, and the latest security state. On startup, the orchestrator hydrates in-memory state from the last 50 records. When the mobile app reconnects, it publishes a replay request and receives a bulk history dump — ensuring the UI is populated even after hours offline.

# 5. Mobile Application

## 5.1 Technology Stack

| Package | Purpose |
| --- | --- |
| Flutter 3.27+ | Cross-platform mobile framework |
| flutter_riverpod | Reactive state management (StreamProviders) |
| mqtt_client | MQTT broker connection, pub/sub |
| fl_chart | Live sensor data visualizations |
| web_socket_channel | Camera JPEG frame streaming |
| flutter_local_notifications | Alert banners when app is open |
| flutter_foreground_task | Android foreground service — keeps MQTT alive when locked |
| flutter_markdown | Renders LLM chat replies with rich text |
| google_fonts | Plus Jakarta Sans (UI) + JetBrains Mono (numbers) |
| shared_preferences | Persists broker host/port between launches |

## 5.2 Application Screens

| Screen | Tab | Key Features |
| --- | --- | --- |
| Dashboard | Home | Animated threat ring (live score, 90s decay), four sensor tiles (motion, sound, temperature, humidity), arm/disarm, buzzer trigger |
| Camera | Camera | Live WebSocket JPEG stream, LIVE/OFFLINE status, fps counter, pinch-to-zoom, fullscreen |
| Reasoning Log | Reasoning | Agent decision cards with timeline: context → LLM reasoning → tool calls → final action |
| Event History | History | Searchable log with All/Today/Week/Critical filters; disarmed state banner |
| Agent Chat | Agent | Conversational MQTT chat with LLM; suggestion chips; markdown rendering; typing indicator |
| Settings | Settings | Broker host/port edit; notification toggles; camera relay config |
| Live Feed | (overlay) | Rolling 1m/5m/10m charts: sound area chart, temperature line, humidity line, motion histogram |

## 5.3 Background Notifications

The app uses flutter_foreground_task to start an Android foreground service on launch. This service maintains an independent MQTT connection in a separate Dart isolate. When the threat level transitions to WARNING or ALERT, flutter_local_notifications fires a high-priority banner notification — even when the phone is locked or the app is in the background. A persistent status bar notification ('SentryAgent — Monitoring…') satisfies Android's requirement for foreground services.

## 5.4 Real-Time State Flow

MqttDataSource subscribes to all agent topics and exposes Dart Stream objects. Riverpod StreamProviders wrap these streams; UI widgets rebuild automatically when new data arrives. Connection status (CONNECTING / CONNECTED / DISCONNECTED) is tracked and displayed via a ConnectionPill widget at the top of the dashboard. On reconnection, a replay request is automatically published to restore full history.

# 6. Communication Protocol (MQTT)

## 6.1 Why MQTT

MQTT (Message Queuing Telemetry Transport) is the ideal protocol for this system. It is lightweight (minimal overhead for battery-constrained devices), supports a publish-subscribe model (decouples producers from consumers), provides QoS levels for reliability, and Mosquitto is a mature, production-grade broker that runs efficiently in Docker. All components — Pi, backend services, and mobile app — use MQTT as the single integration bus.

## 6.2 Topic Contract

| Topic | Publisher | Subscriber | Payload |
| --- | --- | --- | --- |
| iot/pi/telemetry | Raspberry Pi | Agent (Pi bridge) | Combined sensor JSON |
| home/sensors/<type> | Mock sensors | Agent (classifier) | SensorReading JSON |
| home/events | Agent | App | SecurityEvent JSON |
| home/agent/state | Agent (retained) | App | SecurityState JSON |
| home/agent/decision | Agent | App | AgentDecision JSON |
| home/agent/chat/out | Agent | App | ChatMessage JSON |
| home/agent/replay | Agent | App | Bulk history dump |
| home/control/arm | App | Agent | {"armed": true/false} |
| home/control/siren | Agent / App | Agent / Pi | {"action": "trigger"} |
| home/control/chat/in | App | Agent | ChatMessage JSON |
| home/control/replay | App | Agent | {} |

## 6.3 Schema Symmetry

A key design decision is that Python Pydantic models and Dart data classes share identical field names and JSON representations. This eliminates any mapping layer — the same JSON published by the Python agent is decoded directly by the Flutter app with no transformation. For example, ThreatLevel is an enum with values safe, warning, alert in both Python and Dart.

# 7. AI & Machine Learning Integration

## 7.1 Local LLM with Ollama

Ollama is an open-source tool that runs large language models locally on commodity hardware. SentryAgent uses Ollama with the Qwen 2.5 7B Instruct model by default. The model is configured to support native tool calling via the /api/chat endpoint, enabling the agent to iteratively query sensors and events before making a decision. When an NVIDIA GPU is available, Ollama uses CUDA for acceleration; otherwise it falls back to CPU inference. The Docker Compose file exposes GPU resources to the Ollama container automatically.

## 7.2 Two-Layer Detection Strategy

The system uses a two-layer approach to balance speed, cost, and intelligence:

Layer 1 — Rule-based classifier: fast, deterministic, zero latency. Handles 95% of readings (routine sensor updates, clearly safe activity). Runs synchronously on every sensor message.

Layer 2 — LLM reasoning: contextual, tool-augmented, ~2–5 second latency. Invoked only for WARNING and ALERT events. Considers history, armed state, time of day, and tool query results before deciding.

This hybrid approach avoids the cost of calling the LLM for every heartbeat while ensuring that genuinely suspicious events receive intelligent analysis.

## 7.3 Prompt Engineering

Three Jinja2 prompt templates govern LLM behavior:

system.j2 — Defines the agent persona (SentryAgent, a calm security professional), tool schema, and response format constraints.

event_evaluation.j2 — Provides the current security state, the triggering event details, recent event history, and instructions for the decision workflow.

chat.j2 — Configures the conversational persona: concise, plain text, no markdown, 1–3 sentences maximum.

## 7.4 Tool-Calling Workflow

The LLM can call tools iteratively in a loop. For each tool call returned, the orchestrator executes the tool and appends the result as an assistant message. The loop continues until the model returns a final action without a tool call. The final AgentDecision contains: threat_score (0–10), reasoning (plain text explanation), and final_action (one of: ignore, log, notify_user, request_confirmation, trigger_siren, auto_resolve).

# 8. System Integration & Data Flow

## 8.1 Complete Processing Pipeline

Sensor Acquisition: PIR, sound, and temperature sensors on the Pi update every ~1 second. The pi_sensor_publisher.py script reads GPIO and publishes a single combined JSON to iot/pi/telemetry.

Pi Bridge: The orchestrator's _on_pi_telemetry handler receives the combined JSON and fans it out into individual SensorReading objects — one per sensor type.

Classification: EventClassifier.classify() applies rules based on sensor type, value, armed state, and time of day. Boring readings (safe activity, system disarmed) return None and are silently ignored.

Event Storage & Publishing: New SecurityEvent objects are appended to an in-memory deque, persisted to SQLite, and published on home/events for the mobile app.

LLM Queue: Events with severity ≥ WARNING are placed into a bounded asyncio queue (max 10). Overflow is dropped to prevent queue bloat during sensor storms.

Decision Engine: A single async worker processes decisions serially. DecisionEngine.decide() renders the event prompt, calls Ollama, processes tool calls, and returns an AgentDecision.

Decision Publishing: The AgentDecision is persisted to SQLite and published to home/agent/decision. If the action is trigger_siren, a siren command is published.

State Ticker: The orchestrator publishes SecurityState on every telemetry tick. The threat_score is computed live: motion currently detected adds 4, loud sound adds 3, and each recent event contributes a tail that decays linearly over 90 seconds — so the ring spikes the moment something happens and relaxes to zero shortly after the room goes quiet.

Mobile App Update: The Flutter app receives state, event, and decision messages via MQTT StreamProviders. All UI widgets rebuild automatically. Threat level changes trigger haptic feedback and push notifications.

## 8.2 Camera Streaming Pipeline

The Pi's camera streamer connects to the FastAPI camera relay at ws://laptop:8000/ws/camera/stream and pushes binary JPEG frames. The relay's FrameHub holds the latest frame in memory and broadcasts it to all connected viewers at ws://laptop:8000/ws/camera/view. The Flutter CameraScreen connects to the view endpoint and renders each JPEG as an Image.memory widget, achieving ~10 fps live preview. When no Pi is connected, the relay reports status NO SIGNAL and the app displays an appropriate indicator.

# 9. Testing & Evaluation

## 9.1 Automated Backend Tests

The backend includes 74 automated pytest tests organized into modules that cover the complete system without requiring Docker, a running MQTT broker, or Ollama:

| Test Module | Coverage |
| --- | --- |
| test_smoke.py | Module imports, configuration loading, dependency checks |
| test_classifier.py | All classifier rules: motion (armed/disarmed, day/night), sound thresholds, door, temperature |
| test_orchestrator.py | State management, Pi bridge, arm/disarm, event queuing |
| test_storage.py | SQLite CRUD, WAL mode, JSON serialization/deserialization |
| test_mqtt.py | MqttBus handler registration, topic routing |
| test_chat.py | ChatService conversation history, context building |
| test_pi_bridge.py | Combined telemetry parsing, noise threshold mapping |
| test_camera_relay.py | FastAPI WebSocket hub, MJPEG endpoint, frame broadcasting |

## 9.2 Demo Verification Checklist

The following checklist is used to verify the system before a live demo:

Docker containers (broker, agent, camera) are all healthy: docker compose ps

Ollama has downloaded qwen2.5:7b-instruct model

Pi is reachable over SSH and sensor publisher is running

Camera streamer is connected (relay status endpoint returns active)

Mobile app shows CONNECTED pill and receives live sensor data

Arm the system and trigger motion → WARNING/ALERT event appears in app

Agent chat responds within ~5 seconds

Lock phone → trigger alert → notification appears on lock screen

# 10. Results & Demo

## 10.1 System Performance

| Metric | Result |
| --- | --- |
| Sensor-to-event latency (classifier) | < 50 ms |
| Event-to-LLM-decision latency (CPU) | ~3–8 seconds |
| Event-to-LLM-decision latency (GPU) | ~1–3 seconds |
| State update frequency | Every 2 seconds (per Pi telemetry tick) |
| Camera streaming frame rate | ~8-10 fps (JPEG over WebSocket, measured) |
| App MQTT reconnect time | < 2 seconds |
| Backend test suite duration | ~0.5 seconds (74 tests) |
| SQLite history replay on reconnect | < 500 ms (50 records) |

## 10.2 Demo Scenarios

Three scenarios were demonstrated during the university presentation:

Normal Operation (Disarmed): Sensors stream data, all events classified as SAFE. App shows green threat ring. History screen displays informational banner explaining system is disarmed.

Armed + Motion Detection: System armed, PIR triggered at night → ALERT event → LLM evaluates with tool calls → AgentDecision with trigger_siren action → Siren command published → App threat ring turns red, phone buzzes.

Chat Interaction: User asks 'What happened in the last hour?' → ChatService queries SQLite → LLM summarizes events → Plain-text reply rendered in chat bubble.

## 10.3 Key Achievements

Full end-to-end IoT pipeline from physical sensors to mobile UI, running entirely on local network.

LLM with tool-calling capability making autonomous security decisions without cloud API.

Flutter app with live MQTT updates, camera streaming, and lock-screen push notifications.

Two-layer detection correctly classifies 100% of test scenarios with no false positives in demo.

Complete Docker Compose stack with one-command startup (start.ps1 / start.sh).

74 automated backend tests passing in under 1 second.

Continuous sound levels derived from a binary comparator via interrupt pulse-rate mapping, plus a live threat ring with per-event 90-second decay.

Humidity promoted to a first-class sensor: DHT11 read through the Linux kernel IIO driver, with its own dashboard tile and live-feed chart.

# 11. Limitations & Future Work

## 11.1 Current Limitations

| Limitation | Impact | Mitigation |
| --- | --- | --- |
| No MQTT authentication | Any device on LAN can publish/subscribe | Acceptable for home LAN; add username/password for production |
| LLM latency on CPU | 3–8s decision delay on non-GPU hardware | Use GPU or smaller model (3B); rule-based hard-escalation for obvious threats |
| No Firebase push notifications | App must be running for lock-screen alerts | Foreground service implemented; FCM planned for Phase 4 |
| Pi deployment was manual (resolved) | Initial setup required hand-configuration | pi_deploy/ now ships the publisher, camera streamer and setup_pi.sh; a crontab @reboot entry auto-starts everything |
| Single camera, no NVR | No video recording, no playback | Planned: save JPEG sequences to SQLite or filesystem |
| No biometric app lock | Anyone with phone can arm/disarm | Planned: Flutter biometric auth (Phase 4) |

## 11.2 Future Work (Roadmap)

Phase 3 — Hardware Deepening:

Native GPIO sensor module (sentry/sensors/pi.py) to read GPIO directly from within the Docker container.

Pi control topic (iot/pi/control) for LED indicators and siren relay on the Pi side.

Multi-camera support with per-camera relay instances.

Phase 4 — Production Hardening:

Firebase Cloud Messaging (FCM) for true background push notifications.

MQTT username/password authentication and TLS encryption.

Biometric app authentication (fingerprint/face unlock).

Video clip recording triggered by ALERT events.

Geofencing for automatic arm/disarm based on phone location.

# 12. Conclusion

SentryAgent demonstrates that a fully functional, intelligent home security system can be built without any cloud dependency. By combining IoT edge hardware (Raspberry Pi with sensors and camera), a robust Python backend with two-layer AI threat detection, a local LLM capable of contextual reasoning and tool use, and a polished Flutter mobile application, the project achieves all stated objectives.

The two-layer detection strategy — deterministic rules for common cases, LLM reasoning for ambiguous ones — proves effective in practice. The system correctly distinguishes normal household activity from genuine security events based on time of day, armed state, and sensor history. The conversational agent interface provides an intuitive way for users to query system status and understand AI reasoning.

The project also establishes important engineering principles: MQTT as a lightweight integration bus, schema symmetry between backend and frontend models, event-driven reactive UI with Riverpod, and comprehensive automated testing for the AI pipeline. These patterns are directly applicable to production IoT systems at larger scale.

Future work will focus on adding Firebase push notifications, MQTT security, native Pi GPIO integration, and video recording — transforming SentryAgent from a working prototype into a production-quality local security platform.

# References

[1] Eclipse Foundation. (2024). Eclipse Mosquitto MQTT Broker. https://mosquitto.org/

[2] Ollama. (2024). Run Large Language Models Locally. https://ollama.com/

[3] Qwen Team. (2024). Qwen 2.5: A Party of Foundation Models. Alibaba Cloud.

[4] Flutter Team. (2024). Flutter — Build apps for any screen. https://flutter.dev/

[5] Riverpod. (2024). A Reactive Caching and Data-binding Framework. https://riverpod.dev/

[6] FastAPI. (2024). FastAPI — Modern, fast web framework for Python. https://fastapi.tiangolo.com/

[7] Pydantic. (2024). Data validation using Python type hints. https://docs.pydantic.dev/

[8] OASIS Standards. (2019). MQTT Version 5.0 Specification. https://mqtt.org/

[9] Raspberry Pi Foundation. (2024). Raspberry Pi Documentation. https://www.raspberrypi.com/documentation/

[10] Python Software Foundation. (2024). asyncio — Asynchronous I/O. https://docs.python.org/3/library/asyncio.html