SentryAgent

An LLM-Driven Agentic Home Security System

IoT + Mobile Development University Project

# Project Overview

SentryAgent is a home security system whose core innovation is an LLM-powered reasoning agent running on a Raspberry Pi. Unlike traditional rule-based alarms that fire on any sensor trigger, SentryAgent treats security as a decision-making problem: when sensors detect potential threats, an agent equipped with contextual tools reasons about the situation, chooses an appropriate action from a defined toolset, and explains its reasoning back to the user through a Flutter mobile app.

The system separates fast deterministic logic (real-time threat scoring, siren control) from slow intelligent reasoning (agent-based escalation decisions, natural-language summaries). This mirrors how modern production AI systems are built and provides graceful degradation when network connectivity is lost.

# Project Objectives

- Monitor the home environment using four distinct sensors connected to a Raspberry Pi

- Establish two-way MQTT communication between the Pi and the Flutter mobile app

- Implement a fast rule-based threat scoring layer for real-time response

- Build an LLM-powered agent layer that reasons over context and selects actions from a defined toolset

- Provide an in-app conversational interface where users can query the system in natural language

- Send real-time push notifications via Firebase Cloud Messaging (FCM) directly to the mobile app

- Surface the agent's reasoning transparently in the app, so users can see why every action was taken

- Serialize sensor data using JSON, store events in SQLite, and expose a lightweight REST API

- Ensure modular, well-commented code structured into separate Python modules

# Core Innovation: Two-Layer Intelligence

SentryAgent's architecture separates concerns by speed and determinism. This is critical: a security system must respond in milliseconds to clear threats, but should reason carefully about ambiguous ones. A single layer cannot do both.

## Layer 1 — Fast Deterministic Layer (runs on Pi, always available)

- Reads sensors and computes a weighted threat score in real time

- Triggers immediate physical actions (siren, LED) when score exceeds critical threshold

- Logs every event to local SQLite database

- Works offline; never depends on internet or cloud services

## Layer 2 — Intelligent Agent Layer (LLM-powered, ambiguous cases only)

- Activated only when threat score is in the YELLOW zone (ambiguous, score 4–6)

- Receives full context: recent sensor events, time of day, day of week, current arming state, last user activity

- Has access to a defined toolset and chooses which to invoke

- Returns a structured decision plus a natural-language explanation

- Latency of 2–5 seconds is acceptable here because RED-zone threats are already handled by Layer 1

# The Agent's Toolset

The LLM agent is bounded by a strict set of tools. It cannot generate arbitrary code or take arbitrary actions; it can only invoke functions from this list. This makes the system safe and auditable.

| Tool | Purpose |
| --- | --- |
| query_recent_events(hours) | Retrieve sensor events from the last N hours from SQLite |
| get_arming_status() | Check whether the system is currently armed or disarmed |
| send_app_notification(msg, level) | Push a notification to the user's mobile app via FCM |
| request_user_confirmation(msg) | Ask the user 'is this you?' through the app and wait for response |
| trigger_siren(duration_sec) | Activate the physical siren on the Pi |
| log_decision(reasoning) | Persist the agent's reasoning to the database for the app to display |

Example agent flow: PIR detects motion at 2:47 AM. Threat score = 5 (YELLOW). The agent is invoked. It calls get_arming_status() (returns: ARMED), then query_recent_events(2) (returns: door opened 30 seconds ago). It calls request_user_confirmation("Door opened and motion detected at 2:47 AM. Is this you?"). The user has 60 seconds to respond. If no response, the agent calls trigger_siren(30) and send_app_notification("No response to confirmation — alarm triggered", "critical").

# Hardware & Sensors

The system uses four sensors, each contributing distinct signals to the threat scoring layer:

| Sensor | Module | Purpose | Trigger Condition |
| --- | --- | --- | --- |
| PIR Motion Sensor | Grove PIR Motion | Detects movement | Movement in frame |
| Temp/Humidity Sensor | DHT11 | Detects fire/heat risk | Temp > 40°C |
| Sound Sensor | Grove Sound Sensor | Detects glass breaking/loud noise | Above adaptive baseline |
| Door Reed Switch | Magnetic Reed Switch | Detects door/window opening | Magnetic contact broken |

# Threat Scoring Logic (Layer 1)

The fast deterministic layer assigns weighted points to triggered sensors and produces a numerical threat score:

- Motion detected: +3 points

- Sound spike above baseline: +2 points

- Door opened: +2 points

- High temperature: +1 point

- Score 0–3 (GREEN): Safe — log event, no action

- Score 4–6 (YELLOW): Ambiguous — invoke LLM agent for reasoning

- Score 7+ (RED): Critical — trigger siren immediately, notify user, agent generates summary post-hoc

The sound baseline is computed as a rolling average over 24 hours, so the system adapts to each home's ambient noise level. This is the only adaptive component in Layer 1; everything else is deterministic for predictability.

# Flutter Mobile Application

The app is the only user interface — there is no Telegram bot or web dashboard. Every interaction with the system happens through the app, which keeps the user experience cohesive and the architecture clean.

## Screens

- Home Dashboard — Live threat indicator (animated ring, GREEN/YELLOW/RED), current arming state, four sensor tiles showing live values, primary ARM/DISARM button

- Agent Console — Conversational chat interface where users ask the system questions in natural language ("what happened last night?", "is everything okay?"). The agent uses query_recent_events and get_arming_status to answer.

- Reasoning Log — Chronological list of every YELLOW-zone decision the agent made, including full reasoning, tools invoked, and final action. This is the project's transparency feature.

- Alert History — Searchable log of all sensor events with severity badges, timestamps, and links into the Reasoning Log

- Sensor Live Feed — Real-time stream of MQTT messages with charts (fl_chart) showing sound levels, temperature, and event frequency

- Settings — Arm/disarm scheduling, notification preferences, agent confirmation timeout, system status

## Technical Stack

- Flutter + Dart, targeting Android (iOS optional, later)

- mqtt_client package for real-time sensor subscriptions

- firebase_messaging for push notifications (so alerts reach the user even when the app is closed)

- http package for REST queries against the Pi's API

- fl_chart for sensor trend visualization

- provider or riverpod for state management

# Code Architecture (Raspberry Pi)

Every Python module is isolated, independently testable, and well-commented. The clean separation between fast and intelligent layers is visible in the file structure:

| Module | Layer | Responsibility |
| --- | --- | --- |
| sensor_reader.py | Fast | Reads all four sensors at configurable intervals |
| mqtt_client.py | Fast | Publishes sensor data; subscribes to control commands |
| threat_scorer.py | Fast | Weighted threat scoring; deterministic; runs every cycle |
| actuator.py | Fast | Controls siren and LED; called only by threat_scorer |
| agent_engine.py | Intelligent | Builds prompts, calls LLM API, parses tool calls |
| agent_tools.py | Intelligent | Defines and executes the agent's toolset |
| fcm_notifier.py | Intelligent | Sends push notifications to the Flutter app |
| db_handler.py | Shared | SQLite I/O for events and agent decisions |
| serializer.py | Shared | JSON serialization for all payloads |
| api_server.py | Shared | Flask REST API for Flutter app queries |
| main.py | Shared | Orchestrates all modules; the entry point |

# Communication Channels

## MQTT — Real-Time Sensor Stream

- Pi publishes sensor readings to topics: home/security/motion, home/security/sound, home/security/door, home/security/temp

- Flutter app subscribes to all topics for live dashboard updates

- Pi subscribes to home/control/arm and home/control/siren for remote commands

## Firebase Cloud Messaging — Push Notifications

- RED-zone alerts trigger an immediate FCM push to the user's device, even when the app is closed

- Agent confirmation requests (YELLOW zone) are also sent via FCM with action buttons ("It's me" / "Trigger alarm")

- This replaces Telegram entirely; one interface, one app, simpler architecture

## REST API — Historical Queries

- Flutter app fetches paginated event history, agent reasoning logs, and statistics from a Flask server on the Pi

- All responses are JSON; the same serializer.py is used by MQTT and REST

# Database

SQLite runs locally on the Raspberry Pi for zero-configuration persistence and offline capability. Three tables:

- events — All sensor events. Fields: id, timestamp, sensor_type, raw_value, threat_score, zone (GREEN/YELLOW/RED), action_taken

- agent_decisions — Every time the agent runs. Fields: id, timestamp, context_json, tools_called_json, reasoning_text, final_action, latency_ms

- user_responses — Tracks app responses to agent confirmations. Fields: decision_id, response, response_time_ms

This schema lets the Reasoning Log screen show a complete audit trail: what the sensors saw, what the agent reasoned, what tools it called, what the user said, and what the final outcome was.

# LLM Integration Details

- Provider: Claude API (Anthropic) for tool-calling support and reliable structured outputs

- Where it runs: Pi makes the API call. The mobile app never holds the API key. The app only receives the agent's already-decided action and reasoning text.

- When it runs: Only on YELLOW-zone events and on user chat queries. RED-zone alerts do not wait for an LLM.

- Cost: Negligible — sensor payloads are small, calls are infrequent. Estimated under $5 for the entire project.

- Offline behavior: If the LLM API is unreachable, YELLOW events fall back to a conservative default (notify user, do not trigger siren). The system never becomes non-functional due to API outages.

- Prompt engineering: The agent receives a strict system prompt defining its role, available tools, and JSON output schema. Tool calls are validated before execution.

# Why SentryAgent Stands Out

- Real agentic AI, not rebranded if-statements. The LLM actually reasons, picks tools, and explains itself. This is genuinely defensible in a viva.

- Two-layer architecture is academically rich. The separation between deterministic real-time control and probabilistic reasoning is exactly how production AI systems are built.

- Transparency built in. Users see why every decision was made. This addresses the "black box" criticism that plagues AI systems.

- Single mobile interface. No Telegram, no web dashboard. The Flutter app is the entire user-facing system, which is cleaner to demo and easier to grade.

- Demo-friendly. Trigger sensors live; show the agent reasoning on screen; respond to its confirmation request; see the action taken — all in real time during the presentation.

- Graceful degradation. Loses internet? Layer 1 still protects the home. LLM down? System falls back to conservative defaults. Phone off? FCM queues the alert.

- Covers every project requirement. MQTT, AI, JSON, database, mobile dashboard, external APIs, modular code, real-time alerting — all present and meaningfully used.

# Summary

SentryAgent reframes home security as an agentic decision-making problem rather than a stimulus-response alarm. By combining a fast deterministic threat scoring layer with an LLM-powered reasoning agent, the system handles clear threats instantly while reasoning carefully about ambiguous ones. The Flutter mobile app provides the only interface — a live dashboard, a conversational chat with the agent, and a transparent log of every decision the agent has made.

This proposal satisfies every requirement of the IoT + mobile development brief while delivering a project that is technically substantive, academically honest about its AI claims, and genuinely impressive to demo.

SentryAgent  •  IoT + Mobile University Project  •  Raspberry Pi + Flutter + Claude API